A. Mass flow inventory data for marine fuels (Tables S1-S15)

Table S1 | Mass flow inventory data for 1kg MDO

| Input/output | Project | Value |
| --- | --- | --- |
| Input | Crude oil mix | 2.69 kg |
|  | Natural gas mix | 0.073 kg |
|  | Water (desalinated; deionized)-Pre-processing stage | 9.24 kg |
|  | Electricity-Pre-processing stage | 4.78 MJ |
|  | Diesel | 0.00192 kg |
|  | Water (desalinated; deionized) | 2.03 kg |
|  | Electricity | 3.49 MJ |
| Output | MDO | 1 kg |

Table S2 | Mass flow inventory data for 1kg Coal-MeOH

| Input/output | Project | Value |
| --- | --- | --- |
| Input | Hard coal | 8.04 kg |
|  | Natural gas | 0.522 kg |
|  | Electricity-Pre-processing stage | 1.39 MJ |
|  | Diesel | 1.916 kg |
|  | Water (desalinated; deionized)-Pre-processing stage | 3.07 kg |
|  | Coal | 1.57 kg |
|  | Electricity | 0.692 MJ |
|  | Water (desalinated; deionized) | 9.03 kg |
| Output | MeOH | 1 kg |
|  | Ammonium sulphate | 0.002 kg |

Table S3 | Mass flow inventory data for 1kg NG-MeOH

| Input/output | Project | Value |
| --- | --- | --- |
| Input | Hard coal | 0.047 kg |
|  | Crude oil | 0.000811 kg |
|  | Electricity-Pre-processing stage | 5.8 MJ |
|  | Natural gas | 0.498 kg |
|  | Electricity | 2.09 MJ |
|  | Water (desalinated; deionized) | 3.63 kg |
| Output | MeOH | 1 kg |

Table S4 | Mass flow inventory data for 1kg e-MeOH

| Input/output | Project | Value |
| --- | --- | --- |
| Input | CO2 | 1.59 kg |
|  | H2 from electrolysis | 0.22 kg |
|  | Electricity-Pre-processing stage | 44.2 MJ |
|  | Steel | 1.53×10−4 kg |
|  | Water (desalinated; deionized)-Pre-processing stage | 2.319 kg |
|  | Steam | 0.4397 MJ |
|  | Electricity | 0.033 MJ |
| Output | CO2 | 0.0770 kg |
|  | MeOH | 1 kg |
|  | NO2 | 1.78×10−6 kg |
|  | Wastewater | 5.71×10−4 kg |

Table S5 | Mass flow inventory data for 1kg bio-MeOH

| Input/output | Project | Value |
| --- | --- | --- |
| Input | Diesel | 0.00662 kg |
|  | Electricity-Pre-processing stage | 0.076 MJ |
|  | Oxygen | 0.8 kg |
|  | Crude oil | 0.0201 kg |
|  | Gasoline | 0.00376 kg |
|  | Biomass | 1.07 kg |
|  | CO2 | 0.0343 kg |
|  | CO | 0.875 kg |
|  | Water (desalinated; deionized) | 10.8 kg |
|  | Electricity | 0.783 MJ |
| Output | MeOH | 1 kg |

Table S6 | Mass flow inventory data for 1kg Conv-LNG

| Input/output | Project | Value |
| --- | --- | --- |
| Input | Crude oil mix | 0.000811 kg |
|  | Natural gas mix | 1.33 kg |
|  | Electricity | 12.7 MJ |
|  | Electricity-liquefaction stage | 1.22 MJ |
| Output | LNG | 1 kg |

Table S7 | Mass flow inventory data for 1kg e-LNG

| Input/output | Project | Value |
| --- | --- | --- |
| Input | Water (desalinated; deionized) | 6.2 kg |
|  | Electricity-Pre-processing stage | 89.4 MJ |
|  | CO2 from DAC | 2.99 kg |
|  | H2 from electrolysis | 0.547 kg |
|  | Electricity | 4.61 MJ |
|  | Methane | 1.09 kg |
|  | Nitrogen | 0.033 kg |
|  | Butane | 0.0128 kg |
|  | Ethane | 0.115 kg |
|  | Propane | 0.0384 kg |
| Output | LNG | 1 kg |

Table S8 | Mass flow inventory data for 1kg bio-LNG

| Input/output | Project | Value |
| --- | --- | --- |
| Input | Diesel | 0.135 kg |
|  | Electricity-Pre-processing stage | 0.0072 MJ |
|  | Biogas | 23.3 MJ |
|  | Methane | 1.09 kg |
|  | Nitrogen | 0.028 kg |
|  | Butane | 0.0109 kg |
|  | Ethane | 0.0981 kg |
|  | Propane | 0.0237 kg |
|  | Electricity | 0.957 MJ |
| Output | LNG | 1 kg |

Table S9 | Mass flow inventory data for 1kg Conv-LH2

| Input/output | Project | Value |
| --- | --- | --- |
| Input | Hard coal | 0.34 kg |
|  | Electricity-Pre-processing stage | 41.9 MJ |
|  | Natural gas | 3.6 kg |
|  | Water (desalinated; deionized) | 9 kg |
|  | Electricity | 43.2 MJ |
| Output | LH2 | 1 kg |

Table S10 | Mass flow inventory data for 1kg e-LH2

| Input/output | Project | Value |
| --- | --- | --- |
| Input | Electricity-Pre-processing stage | 201 MJ |
|  | Water (desalinated; deionized) | 10 kg |
|  | H2 from electrolysis | 1 kg |
|  | Electricity | 36 MJ |
| Output | LH2 | 1 kg |

Table S11 | Mass flow inventory data for 1kg bio-LH2

| Input/output | Project | Value |
| --- | --- | --- |
| Input | Electricity | 35.72 MJ |
|  | Biomass | 24 kg |
|  | Nitrogen | 0.43 kg |
|  | Electricity-liquefaction stage | 21.5 MJ |
|  | Pyrolysis gas | 14.4 kg |
|  | Water (desalinated; deionized) | 5.4 kg |
| Output | LH2 | 1 kg |

Table S12 | Mass flow inventory data for 1kg Conv-LNH3

| Input/output | Project | Value |
| --- | --- | --- |
| Input | Natural gas | 1.03 kg |
|  | Water (desalinated; deionized)- Pre-processing stage | 2.2 kg |
|  | Electricity-Pre-processing stage | 0.995 MJ |
|  | Diesel | 5.76 kg |
|  | Coal | 9.89 kg |
|  | H2 from coke-oven gas | 0.179 kg |
|  | Nitrogen | 0.808 kg |
|  | Electricity | 36.03 MJ |
|  | Water (desalinated; deionized) | 0.6 kg |
| Output | LNH3 | 1 kg |

Table S13 | Mass flow inventory data for 1kg e-LNH3

| Input/output | Project | Value |
| --- | --- | --- |
| Input | Electricity-Pre-processing stage | 36.492 MJ |
|  | Water (desalinated; deionized) | 1.79 kg |
|  | H2 from electrolysis | 0.179 kg |
|  | N2 from air separator | 0.808 kg |
|  | Electricity | 36 MJ |
|  | Water (desalinated; deionized) | 0.6 kg |
| Output | LNH3 | 1 kg |

Table S14 | Mass flow inventory data for 1kg Biodiesel

| Input/output | Project | Value |
| --- | --- | --- |
| Input | Soybean | 2.193 kg |
|  | Electricity-Pre-processing stage | 6.48 MJ |
|  | Hexane | 8.772 g |
|  | Sulphuric acid | 0.021 g |
|  | Methanol | 0.316 kg |
|  | Electricity | 0.61 MJ |
|  | Sodium Hydroxide | 0.011 kg |
|  | Steam | 5.6 MJ |
|  | Phosphoric acid for glycerine | 3.5 kg |
|  | Water | 1.719 kg |
| Output | Biodiesel* | 1 kg |

Note*: Biodiesel in this study refers to fatty acid methyl ester produced by transesterification of soybean oil (soybean-FAME).

Table S15 | Mass flow inventory data for 1kg HFO

| Input/output | Project | Value |
| --- | --- | --- |
| Input | Crude Oil | 3.4 kg |
|  | Natural Gas | 0.25 kg |
|  | Electricity | 7.5 MJ |
|  | Diesel/Fuel Gas | 0.03 kg |
|  | Water | 6 kg |
| Output | HFO | 1 kg |

B. 1,300 discrete scenarios of parameter combinations across the life cycle

The 1,300 scenarios represent comprehensive combinations of all discrete input parameters across the life cycle (see Fig. S1).

Fig. S1 | Composition of the 1,300 assessment scenarios, defined by combinations of power source, fuel production pathway, ship engine type, operational speed, and ship capacity.